ANALISIS KINERJA MESIN PENDINGIN TERMOAKUSTIK
Oleh

Heldinawati Hanifa Haq

11306144032

ABSTRAK

Telah dilakukan penelitian untuk menyelidiki osilasi suhu pada gelombang stasioner, mengukur frekuensi resonansi pipa (resonator termoakustik) dan menguji sistem termoakustik yang telah berhasil dibuat.
Metode yang digunakan untuk menyelidiki osilasi suhu pada gelombang stasioner adalah mengamati dan mengukur suhu di setiap titik dengan menggunakan sensor LM 35DZ bersamaan dengan mengukur amplitudo pada tiap titik menggunakan mic condensor. Metode yang digunakan untuk mengukur frekuensi resonansi adalah meletakkan mic condensor pada posisi (10,00 ± 0,05) cm di ujung pipa kemudian memvariasi frekuensi pada loudspeaker. Metode yang digunakan untuk menguji sistem termoakustik adalah meletakkan sensor suhu LM 35DZ pada tandon panas dan tandon dingin kemudian sistem dijalankan selama 100 menit.
Hasil dari penyelidikan osilasi suhu pada gelombang stasioner menunjukkan bahwa terdapat beda suhu di posisi perut dan simpul gelombang stasioner yakni 0,2°C pada kondisi sistem tidak terisolasi (tanpa selubung styrofoam) dan 1,3°C untuk kondisi sistem terisolasi (dengan selubung styrofoam). Hasil dari pengukuran frekuensi resonansi pipa menunjukkan bahwa pipa (resonator termoakustik) dengan panjang (80,00 ± 0,05) cm memiliki frekuensi resonansi sebesar 392,8408 Hz dengan letak perut gelombang stasioner berada pada posisi (19,00 ± 0,05) cm. Hasil dari pengujian sistem termoakustik menunjukkan bahwa beda suhu antara tandon panas dan tandon dingin pada suhu ruangan 29°Csetelah sistem dijalankan selama 100 menit adalah 7,22°C.
Kata Kunci: gelombang stasioner,osilasi suhu, frekuensi resonansi, tandon panas,  tandon dingin, termoakustik.

PERFORMANCE ANALYSIS OF THERMOACOUSTIC REFRIGERATOR
By 
Heldinawati Hanifa Haq

11306144032

ABSTRACT

The study has been done to investigate the temperature oscillation in stationary wave, to measure the resonance frequency of the pipe, and to test thermoacoustic system. 

The method used to investigate the temperature oscillation was by measuring the temperature at several points of resonator using LM 35DZ sensor and measuring the amplitude using mic condensor. The method used tomeasure theresonance frequencywas by placing a condensormicat position (10.00 ±0.05) cmfrom theend of the pipeand thenvarying thefrequencyof loudspeaker. The method used totest the thermoacoustic systemwas by placing the temperaturesensorLM35DZat the hotandcoldreservoirsthenthe system was runfor 100 minutes.
The results of investigation showed that there was a temperature diffrence of 0.2°C between the antinodal and nodal of points when the system was not isolated (without styrofoam) and 1.3°C for the isolated system (with styrofoam). The results of measurement of resonance frequency showed that the pipe (thermoacoustic resonator) with length (80,00 ±0,05) cm had a resonance frequency of 392,8408 Hz and the antinodal at the position (19.00 ± 0.05) cm measured from the end of  pipe. The results of the testing thermoacoustic system at room temperature (29°C)  showed that the temperature difference between the hot and cold reservoirs after the system was run for 100 minutes was 7,22°C.

Keywords
: stationary wave, temperature oscillation, resonance frequency, hot reservoir, cold reservoir, thermoacoustics.
vii

