PENGARUH PENAMBAHAN KONSENTRASI LARUTAN SURFAKTAN DISODIUMETHYLENEDIAMINETETRAACETIC SALT (Na2EDTA)TERHADAP TEGANGAN PERMUKAAN DAN VISKOSITAS OLI MESIN PERTAMINA ENDURO 4 STROKE

Oleh:

Andri Nur Priyatno

10306144006

ABSTRAK

Penelitian ini bertujuan untuk mengetahui pengaruh penambahan konsentrasi surfaktan terhadap tegangan permukaan dan viskositas oli mesin Enduro 4 Tak produksi Pertamina dengan klasifikasi SAE 20W-50.

Surfaktan yang digunakan yaitu Na2EDTA (Disodium Ethylenediaminetetraacetic Salt) dengan Konsentrasi yaitu 0,43 M, 0,65M, dan 0,87M. Setiap konsentrasi surfaktan dimasukkan kedalam oli dengan satuan persen volume surfaktan dalam larutan yaitu 2%, 4%, 6%, 8%, 10%, 15%, 20%, 25%, dan 30%. Metode yang digunakan untuk menentukan tegangan permukaan larutan adalah Cincin Du Nouy dan dalam menentukan viskositas larutan adalah metode viskometer Ostwald.

Hasil penelitian oli mesin Enduro 4 Tak murni pada 27oC memiliki nilai tegangan permukaan (103 ± 1) mN/m dan nilai viskositas (175 ± 2) cP. Pengaruh penambahan konsentrasi surfaktan ke dalam oli akan cenderung menaikkan nilai tegangan permukaan oli. Kenaikan nilai tegangan permukaan terbaik, teruji pada penambahan konsentrasi surfaktan 0,65M dengan konsentrasi larutan 30%V yaitu (122± 2) mN/m. Sedangkan pengaruh penambahan konsentrasi surfaktan ke dalam oli, akanmenaikkan nilai viskositas oli. Kenaikan nilai viskositas terbaik, teruji pada penambahan konsentrasi surfaktan 0,65 M dengan konsentrasi larutan 30%V yaitu (444 ± 6) cP.

Kata kunci : Oli Mesin Enduro 4 Tak, Surfaktan, Tegangan Permukaan, Viskositas.

EFFECT OF CONCENTRATION SURFACTANT DISODIUM ETHYLENEDIAMINETETRAACETIC SALT (Na2EDTA) ON THE SURFACE TENSION AND VISCOSITY OF PERTAMINA ENDURO 4 STROKE ENGINE OIL.

By:

Andri Nur Priyatno

10306144006

ABSTRACT

This study aims to determine the effect of adding a surfactant concentration to the surface tension and viscocity of the engine oil. Enduro 4 Stroke engine oil production Pertamina with classification SAE 20W-50.

The surfactant used is Na2EDTA (Disodium Ethylenediaminetetraaceti Salt) with a concentration that is 0.43 M, 0.65M and 0.87M. Each concentration incorporated into the oil with unit volume percent surfactant in solution which is 2%, 4%, 6%, 8%, 10%, 15%, 20%, 25%, and 30%. The method used to determine the surface tension of the solution is the Ring Du Nouy and in the determining the viscosity of the solution is Ostwald Viscometer Method.

The research results show the value of surface tension Enduro 4 Stroke engine oil at 27oC is (103 ± 1) mN/m and viscosity values (175 ± 2) cP. The effect of adding the surfactants concentrations in the oil will tend to raise the value of the surface tension of the oil. The increase in surface tension value best, tested the addition of surfactant concentrations 0,65M solution with a concentration 30% V of  is (122 ± 2) mN/m.While the effect of adding the surfactants concentrations in the oil, will  raise the value of the vicosity of the oil. The increase in the value of the best viscosity, tested the addition of surfactant concentration of 0,65M with a concentration of 30%V is (444± 6) cP.

Keywords: Enduro 4 Stroke Engine Oil, surfactant, Surface Tension, Viscocity.